SECTION 08 80 00 -- GLAZING

1.	Design Requirements:

a.	Insulated glass shall with low-e coating shall be used on all exterior applications.  Argon-fill in insulating units is not required.  Options to be reviewed and approved by Brown Project Manager.

b.	Insulating glass shall have a minimum10-year written warranty guaranteeing the insulated glass will not fog or exhibit any other evidence of seal failure.

c.	Spandrel glass shall have ceramic frit.  Backpainted or film type spandrel coatings are not acceptable.

d.	Safety glass shall be provided in locations as required by Code and must be indicated on the Drawings. Installations shall be reviewed and approved by Brown Project Manager.

e.	Reflective glass shall not be allowed unless otherwise approved by a Brown Project Manager.

f.	Mirrors shall have copper and paint back-coatings to protect the silver surface from tarnishing.  Mirrors shall be clipped and adhered to substrates.

g.	If glass entrances or wall systems are frameless, a form of visual identification shall be required at eye-level.  Identification shall be in the form of etching, graphic decaling or sandblasting as acceptable to the Brown Project Manager.

END OF SECTION